Faculty of Engineering & Technology – Electrical & Computer Engineering Department

First Semester 2022 – 2023

Circuits and electronics lab

ENEE2103

PRELAB EXP.8

Name: Amal Ziad

ID: 1192141

Section: 2

Instructor: Dr. Mohammad Jehad Al Juba

TA: Eng. Yazan Yousef

Date:  9th Dec 2022

Part A: CHARACTERESTICS OF AN N-CHANNEL JFET

Simulation of ID

- Questions

- From your graph, above which values of VDS is ID almost unaffected by VDS when VGS=0?

(Vds,Ids)= (4.5691,16.811m)

- For a given value of VDS , (say 10 V ),do equal changes of VGS cause equal changes of ID?

For a given value of Vds ,changes of Vgs doesn’t equal changes to Id

- Can you measure IG or is it too small?

It’s too small, so it’s very hard to measure it

Part B : COMMON DRAIN AMPLIFIER

- Bias point analysis

Vg= 652.17 mV

Vs=2.331 V

- Simulation of Vout

Av=VoutVin

Av= 0.151/0.2=0.755

-Simulation of phase shift

In phase , phase difference =0 d

-  Calculate Zin

- Simulation of Iin

Zin=VinIin

Zin=0.2/21.194u=9436.6 Ω

- Calculate Zout

- Simulation of Iout

Zout=VoutIout

Zout=0.2/8.936u=22.38 KΩ

Part C: CONSTANT CURRENT SOURCE

Simulation of VL and ID

| RL(KΩ) | VL(V) | ID(mA) |
| --- | --- | --- |
| 0.1 | 1.23 | 12.003 |
| 0.22 | 2.634 | 11.96 |
| 0.33 | 3.95 | 11.93 |
| 0.47 | 5.58 | 11.889 |
| 0.56 | 6.62 | 11.862 |
| 1 | 11.7 | 11.73 |
| 1.5 | 13.44 | 8.9345 |
| 2 | 13.9 | 6.9508 |
| 3 | 14.3 | 4.768 |